CS218 — Programmable & Interoperable Blockchain

Gas Optimisation Report

Project 3 — DAO Governance Contract

# Section 1 — Optimisations Applied

The following five optimisations were applied across DAOGovernance.sol and Treasury.sol. Each replaces a more expensive pattern with a cheaper equivalent. The table below lists all five for completeness. Section 2 presents the full before/after gas measurement for the primary optimisation.

| # | Optimisation Applied | What Replaced | File |
| --- | --- | --- | --- |
| 1 | Description emitted as event log | string description stored in struct (SSTORE) | DAOGovernance.sol |
| 2 | Custom errors (error X()) | require() with string messages | DAOGovernance.sol |
| 3 | calldata for data + description params | memory keyword — causes heap copy | DAOGovernance.sol |
| 4 | Storage vars cached to memory in vote() | Direct p.startTime, p.endTime etc. reads (SLOAD each) | DAOGovernance.sol |
| 5 | Modifier extracted to internal function | Inline modifier body duplicated per call site | Treasury.sol |

# Section 2 — Primary Optimisation: Description as Event Log

## 2.1  The Problem

In the unoptimised contract, the Proposal struct included a string description field stored directly in contract storage. Writing a string to storage triggers the SSTORE opcode. The EVM charges 20,000 gas for writing to a new storage slot. A 500-character description costs approximately 40,000 gas. This cost is paid by every user who creates a proposal — making it the single most expensive avoidable operation in the contract.

## 2.2  Before — Unoptimised Code

The description field was stored inside the Proposal struct and written to contract storage on every createProposal() call:

// BEFORE: description stored in struct — SSTORE on every proposal

struct Proposal {

uint256 id;

uint256 snapshotId;

uint256 startTime;

uint256 endTime;

uint256 yesVotes;

uint256 noVotes;

bool executed;

bool canceled;

address creator;

address target;

bytes data;

string description;  // <-- stored on-chain, costs ~20,000 gas SSTORE

}

function createProposal(

address target,

bytes memory data,        // memory — heap copy

uint256 votingDelay,

uint256 votingPeriod,

string memory description // memory — heap copy

) external returns (uint256) {

// description written to storage here — costs ~20,000 gas

}

## 2.3  After — Optimised Code

The description field was removed from the struct entirely. Instead it is emitted in the ProposalCreated event. The function parameters also changed from memory to calldata to avoid copying bytes and string onto the heap:

// AFTER: description NOT in struct — emitted as event log only

struct Proposal {

uint256 id;

uint256 snapshotId;

uint256 startTime;

uint256 endTime;

uint256 yesVotes;

uint256 noVotes;

address creator;   // packed with bools in same slot

bool executed;

bool canceled;

address target;

bytes data;

// description field removed entirely

}

function createProposal(

address target,

bytes calldata data,        // calldata — no heap copy

uint256 votingDelay,

uint256 votingPeriod,

string calldata description // calldata — no heap copy

) external returns (uint256) {

// description emitted as event — costs ~375 gas

emit ProposalCreated(proposalCount, msg.sender, target,

startTime, endTime, description);

}

## 2.4  Why This Works

Event logs are stored in transaction receipts — not in contract storage. The EVM charges approximately 375 gas to emit a 32-byte event topic, compared to 20,000 gas for writing a new storage slot with SSTORE. The description is still permanently accessible on-chain via event logs — it is simply not queryable by the contract itself at runtime. Since no contract function ever needs to read the description back from storage, this is a lossless optimisation.

This also aligns with the IPFS pattern mandated in the project rubric: the frontend uploads the full proposal text to IPFS, receives a 46-character CID (content identifier), and passes that CID as the description argument. The on-chain CID is the proof of authenticity — anyone can fetch the content from IPFS and verify it matches the hash.

## 2.5  Gas Cost Comparison

| Storage Method | Gas Cost | Accessible How? | Contract Readable? |
| --- | --- | --- | --- |
| string in struct (SSTORE) | ~20,000 gas | Contract storage | Yes |
| Event log — optimised | ~375 gas | Transaction receipts | No (not needed) |
| IPFS CID in event log | ~512 gas | IPFS + receipts | No (not needed) |

# Section 3 — Before / After Gas Numbers

Both reports generated using: forge test --gas-report on an Anvil local node. 27/27 tests passed in both versions. The highlighted row shows the primary optimisation target.

## 3.1  Comparison — DAOGovernance Function Calls

| Function | Before — Avg Gas (Unoptimised) | After — Avg Gas (Optimised) | Gas Saved | Change |
| --- | --- | --- | --- | --- |
| createProposal() | 307,620 | 287,944 | 19,676 | -6.4% |
| vote() | 73,865 | 75,908 | -2,043 | +2.8% |
| executeProposal() | 72,439 | 74,496 | -2,057 | +2.8% |
| cancelProposal() | 28,112 | 28,364 | -252 | ~0% |
| Deployment (one-time) | 1,487,998 | 2,424,303 | -936,305 | +62.9% |

Key result: createProposal() saves 19,676 gas per call — a 6.4% reduction. This directly corresponds to removing the SSTORE for the description field (~20,000 gas). The deployment cost increased by 936,305 gas because custom errors, struct packing, and calldata handling add bytecode. This is a one-time cost paid by the deployer. The optimised contract breaks even after approximately 48 proposals and delivers permanent savings to all users thereafter.

# Section 4 — Full Gas Reports (forge test --gas-report output)

## 4.1  Before — Unoptimised

| Function | Min | Avg | Median | Max | # Calls |
| --- | --- | --- | --- | --- | --- |
| cancelProposal | 24,020 | 28,112 | 30,132 | 30,132 | 5 |
| createProposal | 273,927 | 307,620 | 308,127 | 330,528 | 23 |
| executeProposal | 28,700 | 72,439 | 77,493 | 108,866 | 11 |
| hasVoted | 2,639 | 2,639 | 2,639 | 2,639 | 2 |
| proposalCount | 2,586 | 2,586 | 2,586 | 2,586 | 3 |
| proposals | 27,044 | 27,044 | 27,044 | 27,044 | 5 |
| quorumPercentage | 2,432 | 2,432 | 2,432 | 2,432 | 1 |
| updateQuorum | 23,769 | 30,843 | 26,916 | 45,772 | 4 |
| vote | 26,221 | 73,865 | 72,327 | 89,427 | 30 |

## 4.2  After — Optimised

| Function | Min | Avg | Median | Max | # Calls |
| --- | --- | --- | --- | --- | --- |
| cancelProposal | 23,987 | 28,364 | 30,574 | 30,574 | 5 |
| createProposal | 254,250 | 287,944 | 288,450 | 310,862 | 23 |
| executeProposal | 28,981 | 74,496 | 81,108 | 111,658 | 11 |
| hasVoted | 3,159 | 3,159 | 3,159 | 3,159 | 2 |
| proposalCount | 2,491 | 2,491 | 2,491 | 2,491 | 3 |
| proposals | 26,366 | 26,366 | 26,366 | 26,366 | 5 |
| quorumPercentage | 2,536 | 2,536 | 2,536 | 2,536 | 1 |
| updateQuorum | 24,179 | 34,640 | 27,466 | 59,451 | 4 |
| vote | 30,660 | 75,908 | 74,284 | 91,384 | 30 |